| Ref | Bathroom Guideline for TfNSW - Requirement | VicX Response |
| --- | --- | --- |
| 3.1 | Bathroom types and numbers |  |
| 3.1.1 | Provision of bathrooms at Stations All stations provide the bathroom facilities outlined in this document to customers on the paid-side of the built environment. Bathroom facilities are open and directly accessible to customers as outlined in Section 3.5 | Comply. Bathroom facilities are on the paid side of concourse, on B02 concourse for South and B05 mezz. platform for North. Comply. |
| 3.1.2 | Main Bathroom Types provided at all stations All main bathroom types are self-contained units which contain a toilet, hand-washing and drying facilities and other facilities outlined in sections 3.5 through 3.12. Each station provides customers with a minimum of four bathrooms: - 3 Super-Standard Bathroom (1 Unisex, 1 Male and 1 Female); and - 1 Accessible Bathroom (Unisex). | Comply. Bathroom types are self-contained units. Responses on bathroom facilities are status-ed in respective sections. Comply. 12 bathrooms are provided. 8 at South, 4 at North: Super-Standard; Ambulant *; Accessible. South; 3 Male, 3 Female; 1 Unisex; 1 Unisex. North; 1 Male, 1 Female; 1 Unisex; 1 Unisex. * Ambulant is also a Super-Standard. |
| 3.1.3 | Male Urinal Rooms Provide a Male Urinal room where patronage levels determine that 10 or more super-standard bathrooms are provided for males within an individual toilet bank. | Not Applicable - VicX has <10 Super-Standard Male bathrooms. |
| 3.1.4 | Lift & Change bathroom Central and Circular Quay stations also provide Lift & Change facilities for customers living with quadriplegia and their carers | Not Applicable - Only for Central & CQ stations. |
| 3.1.5 | Bathroom numbers and type Each station provides customers with a minimum of 4 bathrooms: - 3 Super-Standard Bathroom; and - 1 Accessible Bathroom. | Comply. Refer to item 3.1.2. |
| 3.1.6 | Bathroom Gender For banks of up to 12 super-standard bathrooms, the mix of bathroom genders is as follows: | Comply. Refer to item 3.1.2. |
| 3.2 | Trip Planning |  |
| 3.2.1 | Digital Information (Online & Apps) Customers are able to go online and/or use an app to identify what bathroom facilities are provided at specific stations and where those bathroom facilities are located at the station before they leave. | Scope by Sydney Metro. |
| 3.2.2 | Other information channels for customer information Customers are also able to get this information by: - Phoning 131 500 - Calling the station; and - Relevant printed materials such as timetables. | Scope by Sydney Metro. |
| 3.3 | Finding the Bathroom |  |
| 3.3.1 | Easy to find Signage is placed throughout the station environment to enable customers to easily find a bathroom at any stage of their journey by looking up for a sign. A variety of additional wayfinding tools are provided to customers such as station maps at the entry and on the platforms and online apps | Compliant. Directional and Identification signage is provided to the bathrooms. A1 & A0 poster cases are provided at information zones. Installation of station maps & online apps are Sydney Metro scope |
| 3.3.2 | Digital information When at the station, customers are able to locate a bathroom suitable to their needs at the station through mobile and other digital interfaces. | Scope by Sydney Metro |
| 3.3.3 | Information about nearest alternative bathrooms (Optional) Information about the nearest bank of alternative public bathrooms – either within the station area or outside - is provided at all bathroom banks in case of queues or other issues that impact availability (such as cleaning). For accessible bathrooms – details of the nearest accessible bathroom is also provided, including information about whether it is Left-Hand or Right-Hand. | Optional scope not included |
| 3.4 | Bathroom location at station & external area |  |
| 3.4.1 | Bathroom Location Where space in the built environment permits, bathrooms are on the station island (or islands) or side platforms. Where space or heritage restrictions do not permit the required number of bathrooms to be located on the station island (or islands) or side platforms, then: • at least one unisex Super-Standard bathrooms is located on each station island or side-platform; and • the balance of bathrooms are located in the paid area of the concourse. Entrances to all bathrooms should be in a location where people waiting for bathrooms are readily visible by passing pedestrian traffic – more specifically: • by other customers waiting on the platform (for bathrooms on the platform); or • from the gate line (for those on the concourse). Bathrooms should ideally not be accessed from a corridor (anything less than 3 metres wide) for safety reasons. | N/A for Vic X. Not compliant. This is contradictory to the Vic X SWTCs. Comply. Bathroom are on B02 concourse for South and B05 mezz. platform for North: - South: Customers at the gateline will be able to view people waiting for the bathrooms - North: Customers enroute from gateline to platform will be able to view people waiting for the bathrooms This is not a requirement as it is ideal only. |
| 3.4.2 | Privacy Screens (optional) At some stations, privacy screens may be provided where bathrooms are located on the concourse area. Where a privacy screen is provided, it should enable customers to queue in relative privacy while still providing them with sufficient visibility to both feel and be safe. | Optional requirements. For South, bathrooms are on B02 concourse. However, privacy screen are not provided/required as the layout allows customers to queue in relative privacy while still providing them with sufficient visibility to both feel and be safe. For North, bathrooms not located on the concourse. |
| 3.4.3 | External information about each bank of bathrooms At each bank of bathrooms at a station – where ‘bank’ is either an individual bathroom (with no other bathrooms next to it) or multiple bathrooms in the same spot at a station – information is provided to customers about: - The bathroom designation – signage that clearly communicates gender and/or special needs usage - How to gain access to the bathrooms – operating a controlled access entry system (refer to 3.6.1); - The location of the nearest alternative bathrooms – as outlined in section 3.3.3 (Optional). - A braille map showing the location and type of each bathroom in the ‘set’ as well as all of the information above. | - Comply. Signage is provided in accordance to TfNSW wayfinding guide - Comply. The current design is for the door to be provided with sensor to automatically open when someone wants to enter. There is an indicator outside to show if the bathroom is vacant or occupied. Once the person enters, the bathroom can be locked from inside. The indicator outside will change to occupied. To exit, the person can unlock and the door will open. - Optional scope. - Not Compliant - Signage is provided in accordance to TfNSW wayfinding guide. This includes braille identification sign outside the accessible bathroom. Braille map not included in TfNSW Signage guide. |
| 3.4.4 | Digital Information Display external to each indiv. Bathroom The entrance to each individual bathroom clearly displays in digital format (such as an LED Screen): - Whether the bathroom is vacant or occupied through the use of: - the words ‘vacant’ and ‘occupied’, in large text; (essential) ; and - red and green indicators; (optional). - When the bathroom was last cleaned. (optional) - other relevant data such as name of station, date, time. - how to enter the bathroom (e.g. ‘Tap here’) | - Comply. Also refer to item 3.4.3. - Optional, if required by SM/MTS. - Not Compliant - Not included in the TfNSW Guide - N/A Sensor entry so no signage required |
| 3.4.5 | Information for customers living with visual impairment A braille map of each bathroom is provided outside the bathroom for customers living with visual impairment to orient themselves prior to entering. Additionally, digital content delivered through mobile technology: a) The location and type of each bathroom before they enter and lets them know which bathrooms are currently vacant; b) Once they enter – gives them a visual description of the bathroom they have just entered | Not Compliant - Signage is provided in accordance to TfNSW wayfinding guide . This includes braille identification sign. Braille map not included in TfNSW Signage guide. Scope deemed to be by Sydney Metro. |
| 3.4.6 | Priority for accessible bathrooms Accessible bathroom are marked so that priority is given to customers in a wheelchair | Comply. Signage is provided in accordance to TfNSW wayfinding guide . |
| 3.4.7 | Emergency help button An emergency help button is located within close proximity to the bathrooms. For bathrooms located on the concourse, an emergency help button is located at concourse level within 50 metres of the bathrooms. | Comply. Provided in Female Super-standard, Unisex Ambulant and Unisex Accessible bathrooms as per IRS requirements. |
| 3.4.8 | Exterior lighting Exterior lighting is provided using CPTED* principles at all times during hours that customers could be using the station whenever natural lighting is unavailable. | Comply. Lights are provided. |
| 3.4.9 | CCTV Cameras CCTV cameras are located outside bathrooms and positioned so that they can see the faces of everyone who enters the bathroom and detect vandalism, graffiti and other anti-social behaviours. | TSOM scope. |
| 3.5 | Access and Security (Main bathroom) |  |
| 3.5.1 | Gaining access to standard T super-standard bathrooms Customers are able to access our bathrooms at all hours via an electronically enabled controlled access system | Comply. Bathrooms are sensor and open within the opening hours of the station. |
| 3.5.2 | Accessible bathrooms Customers living with disabilities are able to access the Accessible bathrooms at all hours. Accessible bathrooms are fitted with an MLAK key which can be used outside operating hours to access the Accessible bathroom. (Note – an MLAK key is not required to unlock Accessible bathrooms during the standard hours noted above.) | Comply. Bathrooms are open during the opening hours of the station and are accessed via the lifts from the street. Not applicable, access out of hours is via EACS from the street. |
| 3.5.3 | Locks Adequate locks and other security is provided so that the bathrooms can be safely locked at times that they are not accessible to our customers | Not provided. Bathrooms are assumed to be not locked during or outside the standard hour. Please advise if this is required. |
| 3.5.4 | CPTED Crime Prevention through Environmental Design principles are followed. | Comply, all scope by Vic X contractor required due to CPTED has been followed. |
| 3.6 |  |  |
| 3.6.1 | Entering the bathroom Entry to bathrooms is via an electronic, controlled access system and is touch free Entry is only enabled when the bathroom is vacant and when the controlled access system is activated by customer. The controlled access touchpoint is integrated with other external digital information displays outside individual bathrooms and as such any necessary conduits should be provided for in bathroom design. Vacancy and occupation status is clearly visible on the outside of the bathroom | Comply. Electronic door with sensor opening provided, Refer to item 3.4.3. Comply. Vacant/Occupied indicator inside and out, with control activated by the customer inside. Refer to item 3.4.3. N/A No other digital information. Comply. There is an indicator outside the bathroom noting if it is Vacant or Occupied. |
| 3.6.2 | Closing and locking the door The mechanism to close and lock the door is touch free, able to be activated easily and designed to avoid accidental activation. The status of the door lock is clearly visible to the customer | Comply. Also refer to item 3.4.3. Comply. There is an indicator inside the bathroom noting if it is Vacant or Occupied. |
| 3.6.3 | Exiting the bathroom The mechanism to unlock and open the door is the same mechanism as is used to lock the door. It is touch free, able to be activated easily and designed to avoid accidental activation. The status of the door lock is clearly visible to the customer | Comply. Also refer to item 3.4.3. Comply. Also refer to item 3.4.3. |
| 3.6.4 | Emergency access, power outages and malfunctions Emergency access should be allowed by [TO BE DETERMINED BY THE OPERATOR In the event of a power outage, a back-up generator will kick-in to supply power to the bathrooms to enable customers to readily get in and out. In addition, a manual override function is provided for station staff to be able to open the doors in the event of an electrical malfunction | Not Applicable - Requirements are not stated. Comply. There is dual power supply to the station. Comply. Also refer to item 3.4.3. |
| 3.6.5 | Emergency alert for users who overstay Where users have been in the bathroom for longer than 15 minutes then staff will be notified by the digital control monitor so that they may then go to the toilet door and knock to see if the customer requires help | Scope deemed to be by Sydney Metro/TSOM. |
| 3.7 | Internal bathroom experience (Guiding principles) |  |
| 3.7.1 | Cleanliness Customers are provided with bathrooms that they perceive to be clean prior to their arrival at the station and experience as clean when they use them. They perceive our bathrooms to be clean because: - Our bathrooms are odour free; - Floors are dry, paper-free and clean; - The toilet seat is clean and dry; - The toilet bowl is flushed and has no debris; - Walls are bright, light coloured and free from dirt; - All other surfaces are clean and rubbish free; - Garbage bins are sealed; | Scope deemed to be by Sydney Metro. |
| 3.7.2 | Comfort Customers are provided with the following core requirements : - ‘No touch’ facilities – customers are able to use our bathrooms without having to touch any bathroom surfaces with their hands (including at entry and exit); - Hygiene aids – like toilet seat cleaner and toilet seat covers (see section 3.8) - Adequate facilities as outlined in section 3.1 | Comply. Refer relevant sections. Comply. Refer to 3.8. Comply. Refer to 3.1. |
| 3.7.3 | Quick and easy Customers are able to use our bathrooms as quickly as possible so that they can catch their train or get on with their day. Key aspects of our internal design which aid this are: - Fixtures are easy to use and don’t require customers to stop and think about how they use them - Clearly labelled - so customers can readily identify what’s what - Fixtures are located in easy to find locations – so customers don’t waste valuable time locating them - Hand-washing and drying is quick | Note. Comply. Comply. The toilets are labelled. Comply. Comply. |
| 3.7.4 | Other requirements All fixtures are chosen for their: - Durability – materials are durable and resistant to vandalism and neglect so that they last and/or are easily and cheaply replaced or easily repaired; - Anti-graffiti –surfaces of walls, and doors use graffiti resistant materials; - Quality feel – to discourage vandalism; and - Piping to be concealed – all interior water supply and drainage piping connected to fixtures is concealed to protect against contact. Durable materials resistant to human impact are used for any exposed piping which is secured with sturdy fasteners, hangers and supports | Comply. Comply. Doors are cladded with rimex/stainless steel plate and wall tiles are ceramic. Comply. Comply. |
| 3.8 | Cleanliness and hygiene |  |
| 3.8.1 | Extraction fan above toilet to remove odours Exhaust fans are provided above toilets to quickly pull out odours. To conserve electricity, these are set to turn on when the bathrooms is entered and turn off when the bathroom has been vacant for more than xx minutes. Fan noise must be minimised (recommended limit XX). | ArcMac to respond |
| 3.8.2 | Hand-washing and hand-drying facilities are co-located: The hand-washing and hand-drying facilities are co-located so that customers: - can easily shake excess water off their hands in the basin; - can reach directly from the sink – without turning – to reach for a hand-towel; - can drop the paper-towel directly into the rubbish bin below the paper-towel dispenser (see diagram below for example of how this works): | Comply. Not Applicable – Hand dryer are provided in lieu of paper-towel dispenser. Not Applicable – Hand dryer are provided in lieu of paper-towel dispenser. |
| 3.8.3 | Ventilation fan below basin to dry floors Ventilation fans are provided under the sink to blow air onto the floor to dry wet floors quickly. These under-basin vents have an external speed control which can be turned up immediately after they have been cleaned to speed drying time. Fan noise must be minimised (recommended limit XX) | Not Compliant. |
| 3.8.4 | Sealed Garbage Bins that extract rubbish Garbage bin is provided immediately below the paper-towel dispenser and either immediately behind or next to the wash basin to minimise litter and water on the floor. This garbage bin ‘seals’ once rubbish is placed into it so that customers cannot retrieve anything they place into the bin. This also protects them from any unpleasant odours from anything placed in the bin. The bin opening is large enough to take a baby’s nappy. When rubbish is placed into the bin it is suctioned to another central location both for odour protection, security and to ensure that the bin never overflows. Where the design of the station does not allow for the rubbish to be suctioned away, the bin is large enough that it will not overflow when it is not emptied. | Not Applicable – Hand dryer are provided in lieu of paper-towel dispenser. Comply. Comply. Not compliant. SWTC permits no vacuum extraction. |
| 3.8.5 | Sealed Walls and Floors To prevent odours, mould and bacteria, all floors and walls are anti-microbial and completely sealed | Comply. |
| 3.8.6 | Keep it Clean poster There is a small ‘Keep it clean’ or similar poster on the bathroom wall which reminds customers what they should do to keep the bathroom clean and lets them know the consequences of not doing so. For example: | Scope deemed to be by Sydney Metro. |
| 3.8.7 | Digitally enabled cleaning schedule Utilising electronic counting of bathroom patronage as well as problems reported (see 3.8.8), cleaning schedules respond accordingly to prioritise high use or problem facilities. When cleaning is in progress (as activated by cleaning staff - eg. by electronic card), external digital information displays outside individual bathrooms showing: - Cleaning is in progress - That the bathroom is occupied - Where the nearest similar bathroom is Once cleaning mode is electronically activated exhaust vents automatically respond by increasing power/speed to aid in quickly drying the floors after mopping. On completion, cleaning staff deactivate cleaning mode returning the external information display to normal (ie. vacant, last cleaned at XX) and electronic counting of bathroom patronage resumes. | Scope deemed to be by Sydney Metro. |
| 3.8.8 | Report a problem In the event that a customer finds a problem with the bathrooms’ cleanliness or supply of products such as toilet paper or paper towels – they know what to do in order to have this issue rectified. For example, a mobile number they can text to notify cleaning staff that there is an issue that needs fixing in a specific cubicle | Scope deemed to be by Sydney Metro. |
| 3.8.9 | Hygiene aids Customers are provided with Toilet Seat Cleaner and Toilet Seat Covers so that they can feel they are having a hygienic experience when using our bathrooms. | Comply. |
| 3.8.10 | Sanitary bins A ‘touch-free’ sanitary bin is provided between the toilet and the wall in the Female and Unisex super-standard bathrooms and the Unisex accessible bathroom | Spatial for sanitary bin is provided in design. Provision of sanitary bin is deemed to be by Sydney Metro. Ie, bin is typically to be provided by the operator nominated cleaning contractor. |
| 3.8.11 | Music Speakers within the bathroom pipe in classical music to the bathrooms which provides customers with audio privacy and a more relaxing experience. These speakers are also used for emergency announcements. | Speakers are only provided outside the bathrooms by TSOM. Music (if any) by TSOM. |
| 3.9 | Surfaces and lighting |  |
| 3.9.1 | Floors All floors are slip-resistant, antimicrobial and completely sealed. They are of a minimum or superior standard to seamless antimicrobial epoxy terrazzo as this avoids mould and bacteria build up and is easy to clean. | Comply. Terrazzo tiles provided. |
| 3.9.2 | Walls Walls are covered in large tiles that are ideally floor to ceiling. They have butt joints sealed with hairline thin epoxy resin – or material of a similar or better quality. There are no grout joints. Wall panels are light coloured and antimicrobial and are a of a minimum or superior standard to unengineered quartz. | Not Compliant - Grout joints between wall tiles are visible. Comply. Ceramic white tiles (finish – satin, colour – giallo) are provided. |
| 3.9.3 | Baseboard There is a baseboard in a darker solid colour that contrasts with the walls and floor colour in all bathroom areas to aid in navigation for customers who are vision impaired | Comply. 50mm contrast strip tile (colour - nero) is provided |
| 3.9.4 | Ceiling Ceilings are made of materials that absorb the noise. The ceiling is insulated, again to dull sounds from within the bathroom. | Comply. Acoustic is achieved through full height weight to every bathroom. Moiture resistance plasterboard is specified for the bathroom ceiling, |
| 3.9.5 | Counters/backsplash Counters and splashbacks are completely sealed and are built using non porous materials that are of comparable quality to stone or quartz so as to provide a durable, clean and pleasant to look at finish. | Not Applicable – No counter or vanity top. |
| 3.9.6 | Lighting Lighting provided is warm white lighting which creates a welcome and soft environment. Recessed downlights is provided in each bathroom. Natural lighting is also provided where the station design permits this. Occupancy sensors switch off lighting when no one is present. Lighting at the sink is oriented to provide even vertical illumination (from the sides) onto the face, but somewhat softly to avoid glare into the eyes. Mirrors have side-lighting. Lighting from above the mirror supplement the vertical lighting. Vertical lighting is evenly balanced on both sides of the face. | Comply. Comply - Ceiling mounted PIR motion detector provided in every bathroom. Not Compliant - Ceiling light, providing lighting to sink, is aligned to mirror to provide even vertical illumination. Mirrors don’t have side lighting. |
| 3.9.7 | Kid-Designs The unisex Super-Standard bathrooms (see Section 4.1) will employ design features that provide engagement for babies and young children without making adult customers feel that they are in the wrong cubicle. | Not provided. |
| 3.9.8 | Advertising A spot on the wall opposite the toilet is kept free for the placement of advertisements. | Not Compliant - Future provision of advertising are not identified for walls opposite wall toilet. However some elevations are empty and is possible for advertisement placement. |
| 3.10 | Space and Stowage |  |
| 3.10.1 | Hooks Two hooks are provided at different heights for the stowage of coats, shopping bags, handbags backpacks and similar. Each is able to hold a bag of at least 12 Kgs. Signage clearly indicates the maximum weight that the hook can support. The lower hook is 120 cm off the ground and the higher one around 180 cm off the ground. Hooks are a strong metal with secure mounting. There are no sharp edges. Hooks are long enough to hold bags with a strap that is 10cm wide and hook upward to hold a garment. | Comply. Scope deemed to be by Sydney Metro. Comply. Comply. Comply. |
| 3.10.2 | Shelf A shelf is provided adjacent to the basin for customers to place their belongings on while washing and drying. The shelf should be slightly higher than the basin top to remain dry but no higher than the bottom of the mirror. The shelf is minimum 20cm deep and 45cm in length. | Comply. Comply. Not Compliant - Shelves are 15cm deep x 10cm length (AS1428 compliance). |
| 3.11 | Facilities |  |
| 3.11.1 | Toilet placement Toilets should not be set closer than 60 cm from its centre to the side wall. They should be wall hung, without leg support so as to facilitate cleaning. | Comply. Not compliant - Floor mounted WC pan is provided. |
| 3.11.2 | Toilet cistern The toilet cistern should be recessed behind the wall and should be low-flow | Comply. |
| 3.11.3 | Toilet seat Toilet seats are [TO BE DETERMINED BY THE OPERATOR] | Not Applicable – Requirements are not stated. |
| 3.11.4 | Toilet bowl Toilet bowls are made from [TO BE DETERMINED BY THE OPERATOR] | Not Applicable – Requirements are not stated. |
| 3.11.5 | Toilet seat cleaner and covers Customers are provided with liquid toilet seat cleaner by way of an embedded toilet spray that they can spray onto toilet paper to use to clean the seat. They are also provided with a toilet seat cover to cover the seat with which reduces the use of toilet paper to cover the seat with | Comply. Comply. |
| 3.11.6 | Toilet paper dispenser The receptacle for the toilet paper is embedded in the wall and should be large enough that toilet paper can be provided to last between cleaning schedules while making the theft of toilet paper an onerous and time consuming task . Toilet paper supply levels should be digitally monitored and communicated to customers before they enter the bathroom as well as on a station control monitor (that also tracks bathroom cleaning). The toilet paper dispenser is placed on the wall next to the toilet, slightly forward from where the person is sitting and slightly up. It is easy to reach for people of all sizes, shapes and heights. | Not Compliant - Jumbo roll toilet paper dispenser is specified. However, these are not embedded in wall. Not Compliant. Comply. |
| 3.11.7 | Flushing Our toilets are equipped with sensor operated flushing facilities that sense the level of the water and choose the appropriate level of flush to provide. A manual override is also provided to allow the user to manually flush the toilet by pressing the button. The manual override continues to operate in the event of a power outage. No more than 4.5 litres of water is flushed even if the button continues to be held. | Comply. Not provided. Not applicable – No button. |
| 3.11.8 | Toilet Paper [TO BE DETERMINED BY THE OPERATOR] | Not Applicable – Requirements are not stated. |
| 3.12 | Hand-washing and drying area |  |
| 3.12.1 | Basin Basins have a minimum size of 50 cm in length and 40 cm wide and are not flat bottom as they do not effectively allow dirt and debris to be washed away. All basins are installed into vanity tops and located beneath the vanity. All vanity tops have backsplashes | Comply. Not Applicable - No vanity top. |
| 3.12.2 | Tap and soap dispenser An auto-sensor taps and an auto-sensor soap dispenser is provided over the basin to minimise wet floors. The fixtures are reliable and it is easy for customers to figure out how to use them. The water flow is low-flow. | Comply. Comply. |
| 3.12.3 | Soap Foam soap is provided instead of liquid soap as it reduces water usage and discourages theft | Not compliant - Liquid soap is provided based on the design for soap bottle to be concealed within mirror cabinet. |
| 3.12.4 | Shelf A shelf is provided adjacent to the basin (refer to 3.10.2). | Comply. |
| 3.12.5 | Paper towel dispenser The paper towel dispenser is embedded in the wall above or immediately adjacent to the water basin with a large recessed paper towel supply so that it does not need to be topped up regularly. Paper towel supply levels should be digitally monitored and data sent to station control monitor to inform cleaning staff | Not Applicable – Only hand dryer is provided. Not provided. |
| 3.12.6 | Garbage bin (refer to 3.8.4) |  |
| 3.12.7 | Mirror There is a mirror placed over the hand-basin positioned at a height so that people of short to tall statures are able to see their faces in it. Mirrors are covered with a film that can be easily replaced in the event that the mirror is vandalised. Mirrors are tempered so that if broken, they do not result in sharp shards. Edges and back are sealed and protected from any moisture penetration. | Comply. Not Compliant - No film is provided. Laminated glass,and bevelled to all edges. |
| 3.13 | Other specific needs |  |
| 3.13.1 | Sharpies A recessed sharpie bin is provided in the same location as the rubbish bin. [TO BE DETERMINED BY OPERATOR] | Not Compliant - Surfaced-mounted Sharpie bin is provided, but not in the same location as the rubbish bin. |
| 3.13.2 | Baby-changing table A baby change table will be provided in all super-standard bathrooms (see section 4.1). If this is not possible then a baby change table will be provided in all unisex superstandard bathrooms. In the event that no unisex super-standard bathroom is provided, then baby-change tables are provided in both male and female super-standard bathrooms. Durability is a key requirement for the changing table to ensure the safety of infants. Materials used should be easy to clean and antibacterial. [TO BE DETERMINED BY OPERATOR] | Comply, one in all superstandard toilets Cox to confirm |
| 3.13.3 | Grab bars Grab bars are provided in both super-standard and Accessible bathrooms, however the placement varies depending on the toilet type. For Super-Standard bathrooms A horizontal and vertical grab bar are provided in each super-standard bathroom on both the wall immediately: • Next to the toilet; and • Behind the toilet. For Accessible bathrooms Grab bars are provided in accordance with the Australian Standards for Accessible bathrooms. | Comply. Comply - Grab rail placement are provided in accordance to Section 4, including the example layouts. Ie, next to the WC pan but not behind. Comply. |
| 4.1 | Super-standard bathrooms |  |
| 4.1.1 | Super-standard bathrooms Super-standard bathrooms are 2140mm x 1950mm and provide: • Enough space to fit a pram, large pieces of luggage and large sporting equipment such as a bicycle or a surfboard; • Meet the needs of those with mobility issues and short-long term medical conditions – not otherwise catered for by Accessible bathrooms. They are built to dovetail each other so that the centre of the shared wall contains the rubbish bin and paper towels. A diagram is provided on the following page | Not Compliant - 2035mm x 1950mm. SWTC change was provided for this. Not Compliant - Designed as non-dovetail. Layout was previously presented and accepted by Sydney Metro. |
| 4.2 | Accessible Bathrooms |  |
| 4.2.1 | Accessible bathrooms The bathrooms should meet Disability Discrimination Act and Disability Standards for Accessible Public Transport requirements and be a minimum of 1900mm by 2300mm (the 95th percentile of wheelchair sizes). Any station with two accessible bathrooms should provide one left-hand and one right-hand facility. Where any of requirements in this document conflict with the relevant ASA standard, then the ASA standard should be followed unless it is an optional standard only. | Comply. Both are same-hand. The 2 accessible bathrooms are not located in the same area. 1 at South B02 concourse, and 1 at North B05 mezz. platform. N/A |
| 4.3 | Urinal Rooms |  |
| 4.3.1 | Male Urinals | Not Applicable – No urinal rooms at VicX station |
| 4.3.2 | Privacy screens | Not Applicable – No urinal rooms at VicX station |
| 4.4 | Other rooms |  |
| 4.4.1 | Parents Room | Not Applicable – Requirements are not stated. |
| 4.4.2 | Lift & Change Rooms | Not Applicable - Only for Central & CQ stations. |
| 4.4.3 | Animal Assisted Room | Not Applicable – Requirements not stated. |